Water Level Monitoring System

Arduino Uno | I2C LCD | Water Sensor | RGB LED | Buzzer

# 1. Experiment

This experiment demonstrates a water level monitoring system using an Arduino Uno. The system reads water level data from an analog water sensor, displays the current level and raw value on an I2C LCD (8x2), provides a visual indicator using an RGB LED, and triggers a buzzer when the water level reaches HIGH.

# 2. Requirements

## Software

- Arduino IDE (version 1.8 or later)

- LiquidCrystal_I2C library by Frank de Brabander

- USB driver for Arduino Uno (CH340 or ATmega16U2)

## Knowledge

- Basic understanding of Arduino programming

- Familiarity with analog sensors

- Basic electronics (resistors, LEDs, connections)

# 3. Components

| Component | Quantity | Price | Link to buy |
| --- | --- | --- | --- |
| Arduino nano | 1 | 198 | https://robocraze.com/products/nano-development-board-compatible-with-arduino |
| I2C LCD Display | 1 | 109 | https://robocraze.com/products/16x2-lcd-display-blue |
| Water Level Sensor | 1 | 21 | https://robocraze.com/products/high-sensitivity-water-sensor-compatible-with-arduino |
| RGB LED | 1 | 22 | https://robocraze.com/products/rgb-led-module-for-boards-compatible-with-arduino |
| Buzzer | 1 | 38 | https://robu.in/product/5v-passive-buzzer-5pcs/#tab-robu_other_tab |
| Jumper Wires kit | Several | 128 | https://robocraze.com/products/jumper-wire-set-m2m-m2f-f2f-40-pcs-each |
| Breadboard | 1 | 35 | https://robocraze.com/products/half-breadboard |
| USB Cable | 1 | 33 | https://robocraze.com/products/usb-a-b-cable-3-0-cm |
| 18650 battery | 2 | 40 | https://robocraze.com/products/3-7v-1200mah-18650-battery |
| Battery case | 1 | 21 | https://robocraze.com/products/18650-2-cell-holder |
| 16x2 LCD I2C Interface Adapter | 1 | 53 | https://robocraze.com/products/16x2-lcd-i2c-interface-adapter |
|  | Total | 698~800 |  |

# 4. Connectivity

## I2C LCD (4 wires only)

| LCD Pin | Arduino Pin |
| --- | --- |
| GND | Breadboard(-) |
| VCC | Breadboard (+) |
| SDA | A4 |
| SCL | A5 |

## Water Sensor

| Sensor Pin | Arduino Pin |
| --- | --- |
| VCC | Breadboard (+) |
| GND | Breadboard(-) |
| SIG | A0 |

## RGB LED (Common Cathode)

| RGB LED Pin | Arduino Pin |
| --- | --- |
| R (Red) | D2 |
| GND (Common) | GND |
| G (Green) | D3 |
| B (Blue) | D4 |

## Buzzer

| Buzzer Pin | Arduino Pin |
| --- | --- |
| + (Positive) | D8 |
| - (Negative) | Breadboard(-) |

## Power Connection

| Battery case | Arduino Pin |
| --- | --- |
| + (Positive) | Vin |
| - (Negative) | GND |

## Power Connection

| Arduino nano | Breadboard |
| --- | --- |
| 5V | Breadboard (+) |
| GND | Breadboard (-) |

# 5. Steps Procedure

- Install Arduino IDE from https://www.arduino.cc/en/software

- Open Arduino IDE and go to Tools > Manage Libraries

- Search for LiquidCrystal_I2C by Frank de Brabander and install it

- Connect all components as described in the Connectivity section above

- Connect Arduino Uno to your computer via USB cable

- Go to Tools > Board and select Arduino Uno

- Go to Tools > Port and select the correct COM port

- Copy the code from Section 6 into Arduino IDE

- Click Upload button to upload the code

- Open Serial Monitor (9600 baud) to see raw sensor values

- Dip the water sensor into water and observe the LCD, RGB LED and buzzer

- If LCD shows black boxes, slowly turn the contrast pot on the I2C backpack with a small screwdriver

- Calibrate DRY_VAL and WET_VAL if needed based on Serial Monitor readings

# 6. Coding

## Behaviour Logic

| Water Level | LCD Row 1 | RGB LED | Buzzer |
| --- | --- | --- | --- |
| 0 (no water) | EMPTY | Red | OFF |
| 1 - 250 | LOW | Red | OFF |
| 350 - 510 | MEDIUM | Blue | OFF |
| 600 (full) | HIGH | Green | ON |

## Source Code

#include <LiquidCrystal_I2C.h>

LiquidCrystal_I2C lcd(0x27, 8, 2);  // 8 columns, 2 rows

#define BUZZER  8   // buzzer on D8

#define RED     2   // RGB Red   on D2

#define GREEN   3  // RGB Green on D3

#define BLUE    4   // RGB Blue  on D4

void setRGB(bool r, bool g, bool b) {

digitalWrite(RED,   r);

digitalWrite(GREEN, g);

digitalWrite(BLUE,  b);

}

void setup() {

Serial.begin(9600);

lcd.init();

lcd.backlight();

pinMode(BUZZER, OUTPUT);

pinMode(RED,    OUTPUT);

pinMode(GREEN,  OUTPUT);

pinMode(BLUE,   OUTPUT);

digitalWrite(BUZZER, LOW);

setRGB(0, 0, 0);

}

void loop() {

int value = analogRead(A0);

lcd.setCursor(0, 0);

lcd.print("V:");

lcd.print(value);

lcd.print("    ");

lcd.setCursor(0, 1);

if (value == 0) {

lcd.print("EMPTY   ");

setRGB(1, 0, 0);

digitalWrite(BUZZER, LOW);

} else if (value > 1 && value < 250) {

lcd.print("LOW     ");

setRGB(1, 0, 0);

digitalWrite(BUZZER, LOW);

} else if (value >= 350 && value < 510) {

lcd.print("MEDIUM  ");

setRGB(0, 0, 1);

digitalWrite(BUZZER, LOW);

} else if (value >= 600) {

lcd.print("HIGH    ");

setRGB(0, 1, 0);

digitalWrite(BUZZER, HIGH);

}

Serial.println(value);

delay(300);

}

Circuit diagram